- 1 -

The M-code interpreter

----------------------

Appendix 1 of the yellow report:

N.Wirth.   The   Personal   Computer   Lilith.   Institut  fur

Informatik, ETH. Report 40, 1981.

The following Modula-2 program  interprets  M-code  instructions

and  serves  as a high-level definition of the Lilith computer's

instruction set and architecture. A few comments  are  necessary

to cover details that are not fully described by the program.

1.  The  array  variables  stk  and  code stand for the data and

program  stores  respectively.  We  assume  that  on  an  actual

computer  they  represent  the  SAME  physical memory. The array

indices  then  denote  memory  addresses.  Access  to  the  code

involves  the  use  of  the  base  address F (and an 18-bit wide

addition).

2. All checks against arithmetic overflow, storage overflow, and

access  with  value  NIL  are  omitted  from  the program in the

interest of clarity and in order not to obscure  the  essentials

of the interpretation algorithm.

3.  Certain  instructions are explained in loose English instead

of precise Modula statements. Among them are the bitmap handling

instructions,   which   actually   constitute   fairly   complex

algorithms,  and  also  operations  like  shifts,  packing,  and

unpacking,  which are considered as primitives, and hence not to

be defined contortiously in terms of even lower primitives.

4. The functions low(d), high(d), and pair(a,b)  are  introduced

to  denote selection of a part of a double word and construction

of a  double  word.  The  functions  Dtrunc  and  Dfloat  denote

conversion  of  floating-point  values into double word integers

and vice-versa. All these functions are NOT available in Modula-

2.  Also,  sets  of  the  form  {m..n} are used, although proper

Modula-2 does not  allow  expressions  to  be  used  within  set

constructors.

5. The detailed specification of I/O instructions is suppressed.

It  is  considered  not  to  be  part  of  the  general   M-code

definition,  but  should  be  allowed  to  vary  among different

implementations according to the  available  hardware.  This  is

particularly  true  for  the instructions DSKR, DSKW, SETRK used

for accessing the disk.

6. The interrupt mechanism is described in a rather loose manner

and  requires  additional  explanation:  At  the  start  of each

interpretation  cycle,  the  Boolean  variable  REQ   determines

whether  or  not  an  interrupt  request should be honored.  REQ

means "at least one of the unmasked  interrupt  lines  (numbered

- 2 -

8...15)  is  low".  If  we  denote  the request lines by the set

variable  ReqLines  and  the  presence of a request on line i by

"NOT (i IN ReqLines)", then REQ can be expressed as

REQ = (ReqLines + Mask # {8 .. 15})

The value Mask is the  union  of  the  mask  register  M  and  a

variable  called  DevMask  (Mask  =  M  +  DevMask). This global

variable allows a program (typically the  operating  system)  to

shut  out  any (or all) devices from interrupting. In the Lilith

computer, DevMask is allocated in main memory at location 3. The

value ReqNo determines the interrupt line whose request is being

accepted. It determines the transfer vector used by the TRANSFER

operation. The value ReqNo is defined as the maximum i such that

"NOT (i IN ReqLines + Mask).

Table of instructions

---------------------

0      40     100     140     200     240     300     340

0   LI0    LLW     LGW     LSW0    LSW     READ    FOR1    MOV

1   LI1    LLD     LGD     LSW1    LSD     WRITE   FOR2    CMP

2   LI2    LEW     LGW2    LSW2    LSD0    DSKR    ENTC    DDT

3   LI3    LED     LGW3    LSW3    LXFW    DSKW    EXC     REPL

4   LI4    LLW4    LGW4    LSW4    LSTA    SETRK   TRAP    BBLT

5   LI5    LLW5    LGW5    LSW5    LXB     UCHK    CHK     DCH

6   LI6    LLW6    LGW6    LSW6    LXW             CHKZ    UNPK

7   LI7    LLW7    LGW7    LSW7    LXD     SYS     CHKS    PACK

10   LI8    LLW8    LGW8    LSW8    DADD    ENTP    EQL     GB

11   LI9    LLW9    LGW9    LSW9    DSUB    EXP     NEQ     GB1

12   LI10   LLW10   LGW10   LSW10   DMUL    ULSS    LSS     ALOC

13   LI11   LLW11   LGW11   LSW11   DDIV    ULEQ    LEQ     ENTR

14   LI12   LLW12   LGW12   LSW12           UGTR    GTR     RTN

15   LI13   LLW13   LGW13   LSW13           UGEQ    GEQ     CX

16   LI14   LLW14   LGW14   LSW14   DSHL    TRA     ABS     CI

17   LI15   LLW15   LGW15   LSW15   DSHR    RDS     NEG     CF

20   LIB    SLW     SGW     SSW0    SSW     LODFW   OR      CL

21          SLD     SGD     SSW1    SSD     LODFD   XOR     CL1

22   LIW    SEW     SGW2    SSW2    SSD0    STORE   AND     CL2

23   LID    SED     SGW3    SSW3    SXFW    STOFV   COM     CL3

24   LLA    SLW4    SGW4    SSW4    TS      STOT    IN      CL4

25   LGA    SLW5    SGW5    SSW5    SXB     COPT    LIN     CL5

26   LSA    SLW6    SGW6    SSW6    SXW     DECS    MSK     CL6

27   LEA    SLW7    SGW7    SSW7    SXD     PCOP    NOT     CL7

30   JPC    SLW8    SGW8    SSW8    FADD    UADD    ADD     CL8

31   JP     SLW9    SGW9    SSW9    FSUB    USUB    SUB     CL9

32   JPFC   SLW10   SGW10   SSW10   FMUL    UMUL    MUL     CL11

33   JPF    SLW11   SGW11   SSW11   FDIV    UDIV    DIV     CL10

- 3 -

34   JPBC   SLW12   SGW12   SSW12   FCMP    UMOD            CL12

35   JPB    SLW13   SGW13   SSW13   FABS    ROR     BIT     CL13

36   ORJP   SLW14   SGW14   SSW14   FNEG    SHL     NOP     CL14

37   ANDJP  SLW15   SGW15   SSW15   FFCT    SHR     MOVF    CL15

Reserved locations:

0   (F-register of module 0)

1   (initialization flag of module 0)

2   (string pointer of module 0)

3   device mask

4   P-register

5   saved P-register

6   boot flag

16,17   trap vector

20,21   interrupt vector for line 8 (clock)

22,23   interrupt vector for line 9 (disk)

...

36,37   interrupt vector for line 15

40..177 data frame table

MODULE Interpreter;  (*N.Wirth, Ch.Jacobi; Feb.81*)

CONST tlc      = 16B;            (*trap location adr*)

dft        = 40B;            (*data frame table adr*)

VAR  (*global state variables*)

PC:  CARDINAL;               (*program counter*)

IR:  CARDINAL;               (*instruction register*)

F:   CARDINAL;               (*code frame base address*)

G:   CARDINAL;               (*data frame base address*)

H:   CARDINAL;               (*stack limit address*)

L:   CARDINAL;               (*local segment address*)

S:   CARDINAL;               (*stack pointer*)

P:   CARDINAL;               (*process base address*)

M:   BITSET;                 (*process interrupt mask*)

REQ: BOOLEAN;                (*interrupt request*)

ReqNo: CARDINAL;             (*request number, 8..15*)

(*auxiliary variables used over single instructions only*)

i, j, k: CARDINAL;

sz, adr, low, hi: CARDINAL; (*used in FOR, ENTP, PCOP*)

sb, db, sbmd, dbmd, fo: CARDINAL;  (*display handling*)

x, y: REAL;

stk:  ARRAY [0..177777B] OF CARDINAL;    (*data store*)

MODULE InstructionFetch;

IMPORT F,PC;

EXPORT next, next2;

- 4 -

VAR code: ARRAY [0..77777B] OF [0..255];

PROCEDURE next(): CARDINAL;

BEGIN

INC(PC); RETURN code[4*F+PC-1]

END next;

PROCEDURE next2(): CARDINAL; (*get next two code bytes*)

BEGIN

INC(PC, 2); RETURN code[4*F+PC-2]*400B + code[4*F+PC-1]

END next2;

END InstructionFetch;

MODULE ExpressionStack;

EXPORT push, pop, Dpush, Dpop, empty;

VAR sp: CARDINAL;

a: ARRAY [0..15] OF CARDINAL;   (*expression stack*)

PROCEDURE push(x: CARDINAL);

BEGIN a[sp] := x; INC(sp)

END push;

PROCEDURE pop(): CARDINAL;

BEGIN DEC(sp); RETURN(a[sp])

END pop;

PROCEDURE Dpush(d: REAL);

BEGIN a[sp] := high(d); INC(sp); a[sp] := low(d); INC(sp)

END Dpush;

PROCEDURE Dpop(): REAL;

BEGIN DEC(sp,2); RETURN pair(a[sp], a[sp+1])

END Dpop;

PROCEDURE empty():BOOLEAN;

BEGIN RETURN sp = 0

END empty;

BEGIN sp := 0;

END ExpressionStack;

PROCEDURE mark(x: CARDINAL; external: BOOLEAN);

VAR i: CARDINAL;

BEGIN i := S;

stk[S] := x; INC(S);  (*static link*)

stk[S] := L; INC(S);  (*dynamic link*)

IF external THEN

stk[S] := PC+100000B ELSE stk[S] := PC

END ;

INC(S,2); L := i

END mark;

- 5 -

PROCEDURE saveExpStack;

VAR c: CARDINAL;

BEGIN c := 0; (*expression stack counter*)

WHILE NOT empty() DO

stk[S] := pop(); INC(S); INC(c);

END ;

stk[S] := c; INC(S)

END saveExpStack;

PROCEDURE restoreExpStack;

VAR c: CARDINAL;

BEGIN DEC(S); c := stk[S];

WHILE c>0 DO

DEC(c); DEC(S); push(stk[S])

END

END restoreExpStack;

PROCEDURE saveRegs;

BEGIN saveExpStack;

stk[P  ] := G;  stk[P+1] := L;

stk[P+2] := PC; stk[P+3] := CARDINAL(M);

stk[P+4] := S;  stk[P+5] := H+24;

(* stk[P+6] is reserved for error code *)

(* stk[P+7] is reserved for error trap mask *)

END saveRegs;

PROCEDURE restoreRegs(changeMask: BOOLEAN);

BEGIN

G := stk[P];   F := stk[G];

L := stk[P+1]; PC := stk[P+2];

IF changeMask THEN M := BITSET(stk[P+3]) END ;

S := stk[P+4]; H := stk[P+5]-24;

restoreExpStack

END restoreRegs;

PROCEDURE Transfer(changeMask: BOOLEAN; to, from: CARDINAL);

VAR j: CARDINAL;

BEGIN

j := stk[to]; saveRegs; stk[from] := P;

P := j; restoreRegs(changeMask)

END Transfer;

PROCEDURE Trap(n: CARDINAL);

BEGIN

IF NOT (n IN BITSET(stk[P+7])) THEN

stk[P+6] := n;

Transfer(TRUE, tlc, tlc+1)

END

END Trap;

BEGIN (* readBootFile *)

P := stk[4]; restoreRegs(TRUE);

- 6 -

LOOP

IF REQ THEN Transfer(TRUE, 2*ReqNo, 2*ReqNo+1) END ;

IR := next();

CASE IR OF

0B .. 17B: (*LI0 - LI15 load immediate*)  push(IR MOD 16) |

20B: (*LIB  load immediate byte*)  push(next()) |

22B: (*LIW  load immediate word*)  push(next2()) |

23B: (*LID  load immediate double word*)

push(next2()); push(next2()) |

24B: (*LLA  load local address*)   push(L+next()) |

25B: (*LGA  load global address*)  push(G+next()) |

26B: (*LSA  load stack address*)   push(pop()+next()) |

27B: (*LEA  load external address*)

push(stk[dft+next()]+next()) |

30B: (*JPC  jump conditional*)

IF pop() = 0 THEN PC := PC + next2()

ELSE INC(PC,2)

END  |

31B: (*JP   jump*)  PC := PC + next2() |

32B: (*JPFC  jump forward conditional*)

IF pop() = 0 THEN PC := PC + next() ELSE INC(PC) END  |

33B: (*JPF  jump forward*)   PC := PC + next()  |

34B: (*JPBC  jump backward conditional*)

IF pop() = 0 THEN PC := PC - next() ELSE INC(PC) END  |

35B: (*JPB  jump backward*)  PC := PC - next()  |

36B: (*ORJP  short circuit OR *)

IF pop() = 0 THEN INC(PC)

ELSE push(1); PC := PC+next()

END |

37B: (*ANDJP  short circuit AND *)

IF pop() = 0 THEN push(0); PC := PC+next()

ELSE INC(PC)

END |

40B: (*LLW  load local word*)     push(stk[L+next()])  |

41B: (*LLD  load local double word*)

i := L+next(); push(stk[i]); push(stk[i+1])  |

- 7 -

42B: (*LEW  load external word*)

push(stk[stk[dft+next()]+next()]) |

43B: (*LED  load external double word *)

i := stk[dft+next()]+next();

push(stk[i]); push(stk[i+1]) |

44B .. 57B: (*LLW4-LLW15*)   push(stk[L + (IR MOD 16)])  |

60B: (*SLW  store local word*)    stk[L+next()] := pop() |

61B: (*SLD  store local double word*)

i := L+next(); stk[i+1] := pop(); stk[i] := pop()  |

62B: (*SEW  store external word*)

stk[stk[dft+next()]+next()] := pop() |

63B: (*SED  store external double word *)

i := stk[dft+next()]+next();

stk[i+1] := pop(); stk[i] := pop() |

64B .. 77B: (*SLW4-SLW15  store local word*)

stk[L+(IR MOD 16)] := pop()  |

100B: (*LGW  load global word*)    push(stk[G+next()]) |

101B: (*LGD  load global double word*)

i := next()+G; push(stk[i]); push(stk[i+1]) |

102B .. 117B: (*LGW2 - LGW15  load global word*)

push(stk[G + (IR MOD 16)]) |

120B: (*SGW  store global word*)   stk[G+next()] := pop() |

121B: (*SGD  store global double word*)

i := G+next(); stk[i+1] := pop(); stk[i] := pop()  |

122B .. 137B: (*SGW2 - SGW15  store global word*)

stk[G + (IR MOD 16)] := pop() |

140B .. 157B: (*LSW0 - LSW15  load stack addressed word*)

push(stk[pop()+(IR MOD 16)]) |

160B .. 177B:  (*SSW0 - SSW15  store stack-addressed word*)

k := pop(); i := pop()+(IR MOD 16); stk[i] := k |

200B: (*LSW  load stack word*)

i := pop() + next(); push(stk[i]) |

201B: (*LSD  load stack double word*)

i := pop() + next(); push(stk[i]); push(stk[i+1]) |

- 8 -

203B: (*LXFW  load indexed frame word*)

k := pop() + pop()*4; push(stk[k]) |

202B: (*LSD0  load stack double word*)

i := pop(); push(stk[i]); push(stk[i+1]) |

204B: (*LSTA  load string address *)  push(stk[G+2]+next()) |

205B: (*LXB  load indexed byte*)

i := pop(); j := pop(); k := stk[j + (i DIV 2)];

IF i MOD 2 = 0 THEN push(k DIV 400B)

ELSE push(k MOD 400B)

END |

206B: (*LXW  load indexed word*)

i := pop()+pop(); push(stk[i]) |

207B: (*LXD  load indexed double word *)

i := 2*pop()+pop(); push(stk[i]); push(stk[i+1])  |

210B: (*DADD  double add.  Subsequent operators for double

words denote unsigned fixed-point arithmetic,

although the program shows REAL operands*)

y := Dpop(); x := Dpop(); Dpush(x+y) |

211B: (*DSUB  double subtract*)

y := Dpop(); x := Dpop(); Dpush(x-y) |

212B: (*DMUL  double multiply*)

j := pop(); i := pop(); (* x := i*j *) Dpush(x) |

213B: (*DDIV  double divide*)

j := pop(); x := Dpop();

(* k := x DIV j; i := x MOD j *) push(i); push(k) |

216B: (*DSHL  double shift left*)

x := Dpop(); (*shift x left 1 bit*) Dpush(x) |

217B: (*DSHR  double shift right*)

x := Dpop(); (*shift x right 1 bit*) Dpush(x) |

220B: (*SSW  store stack word*)

k := pop(); i := pop()+next(); stk[i] := k |

221B: (*SSD  store stack double word*)

k := pop(); j := pop(); i := pop()+next();

stk[i] := j; stk[i+1] := k |

222B: (*SSD0  store stack double word*)

k := pop(); j := pop(); i := pop();

stk[i] := j; stk[i+1] := k |

223B: (*SXFW  store indexed frame word*)

- 9 -

i := pop(); k := pop() + pop()*4; stk[k] := i |

224B: (*TS  test and set*)

i := pop(); push(stk[i]); stk[i] := 1 |

225B: (*SXB  store indxed byte*)

k := pop(); i := pop(); j := pop() + (i DIV 2);

IF i MOD 2 = 0 THEN

stk[j] := k*400B + (stk[j] MOD 400B)

ELSE stk[j] := (stk[j] DIV 400B) * 400B + k

END |

226B: (*SXW  store indexed word*)

k := pop(); i := pop()+pop(); stk[i] := k |

227B: (*SXD  store indexed double word*)

k := pop(); j := pop(); i := 2*pop()+pop();

stk[i] := j; stk[i+1] := k |

230B: (*FADD  floating add*)

y := Dpop(); x := Dpop(); Dpush(x+y) |

231B: (*FSUB  floating subtract*)

y := Dpop(); x := Dpop(); Dpush(x-y) |

232B: (*FMUL  floating multiply*)

y := Dpop(); x := Dpop(); Dpush(x*y) |

233B: (*FDIV  floating divide*)

y := Dpop(); x := Dpop(); Dpush(x/y) |

234B: (*FCMP  floating compare*)

x := Dpop(); y := Dpop();

IF x > y THEN push(0); push(1)

ELSIF x < y THEN push(1); push(0)

ELSE push(0); push(0)

END |

235B: (*FABS  floating absolute value*) Dpush(ABS(Dpop())) |

236B: (*FNEG  floating negative*)  Dpush(-Dpop()) |

237B: (*FFCT  floating functions*)  i := next();

IF i=0 THEN Dpush(FLOAT(pop()))

ELSIF i=1 THEN Dpush(DFloat(Dpop()))

ELSIF i=2 THEN push(TRUNC(Dpop()))

ELSIF i=3 THEN Dpush(Dtrunc(Dpop(), pop()))

END |

240B: (*READ*)   i := pop(); k := pop();

(* stk[i] := input from channel k *) |

241B: (*WRITE*)  i := pop(); k := pop();

- 10 -

(* output i to channel k *) |

242B: (*DSKR  disk read*) |

243B: (*DSKW  disk write*) |

244B: (*SETRK  set disk track*) |

245B: (*UCHK*) k := pop(); j := pop(); i := pop(); push(i);

IF (i < j) OR (i > k) THEN Trap(4) END |

247B: (*SYS  rarely used system functions*) |

250B: (*ENTP  entry priority*)

stk[L+3] := CARDINAL(M); M := {0..next()-1} |

251B: (*EXP  exit priority*)   M := BITSET(stk[L+3]) |

252B: (*ULSS*) j := pop(); i := pop();

IF i < j THEN push(1) ELSE push(0) END  |

253B: (*ULEQ*) j := pop(); i := pop();

IF i <= j THEN push(1) ELSE push(0) END |

254B: (*UGTR*) j := pop(); i := pop();

IF i > j THEN push(1) ELSE push(0) END  |

255B: (*UGEQ*) j := pop(); i := pop();

IF i >= j THEN push(1) ELSE push(0) END |

256B: (*TRA  coroutine transfer*)

Transfer(BOOLEAN(next()), pop(), pop()) |

257B: (*RDS  read string*)   k := pop(); i := next();

REPEAT

stk[k] := next2(); INC(k); DEC(i)

UNTIL i < 0  |

260B: (*LODFW  reload stack after function return*)

i := pop(); restoreExpStack; push(i) |

261B: (*LODFD  reload stack after function return*)

i := pop(); j := pop(); restoreExpStack;

push(j); push(i) |

262B: (*STORE*)  saveExpStack |

263B: (*STOFV  store stack with formal procedure on top*)

i := pop(); saveExpStack; stk[S] := i; INC(S) |

264B: (*STOT  copy from stack to procedure stack*)

stk[S] := pop(); INC(S) |

- 11 -

265B: (*COPT  copy element on top of expression stack*)

i := pop(); push(i); push(i) |

266B: (*DECS  decrement stackpointer*)  DEC(S) |

267B: (*PCOP  allocation and copy of value parameter *)

stk[L+next()] := S;

sz := pop(); k := S+sz; adr := pop();

WHILE sz>0 DO

stk[S] := stk[adr]; INC(S); INC(adr); DEC(sz)

END |

270B: (*UADD*)  j := pop(); i := pop(); push(i+j) |

271B: (*USUB*)  j := pop(); i := pop(); push(i-j) |

272B: (*UMUL*)  j := pop(); i := pop(); push(i*j) |

273B: (*UDIV*)  j := pop(); i := pop(); push(i DIV j) |

274B: (*UMOD*)  j := pop(); i := pop(); push(i MOD j) |

275B: (*ROR*)   j := pop(); i := pop() MOD 16;

(* k := j rightrotated by i places *) push(k) |

276B: (*SHL*)   j := pop(); i := pop() MOD 16;

(* k := j left shifted by i places *) push(k) |

277B: (*SHR*)   j := pop(); i := pop() MOD 16;

(* k := j right shifted by i places*) push(k) |

300B: (*FOR1  enter FOR statement *)

i := next(); (* =0: up; >0: down *)

hi := pop(); low := pop(); adr := pop();

k := PC + next2();

IF ((i = 0)  AND (low <= hi)) OR

((i # 0)  AND (low >= hi)) THEN

stk[adr] := low;

stk[S] := adr; INC(S); stk[S] := hi; INC(S)

ELSE (* don't execute the FOR loop *)

PC := k

END |

301B: (*FOR2  exit FOR statement *)

hi := stk[S-1]; adr := stk[S-2];

sz := INTEGER(next()); (* step range -128..+127 *)

k := PC + next2(); i := stk[adr]+sz;

IF ((sz >= 0) AND (i > hi))

OR ((sz <= 0) AND (i < hi))

THEN (* terminate *) DEC(S,2)

ELSE (* continue *) stk[adr] := i; PC := k

END |

- 12 -

302B: (*ENTC  enter CASE statement*)

PC := PC + next2(); k := pop();

low := next2(); hi := next2();

stk[S] := PC + 2*(hi-low) + 4; INC(S);

IF (k >= low) AND (k <= hi) THEN

PC := PC + 2*(k-low+1)

END;

PC := PC + next2() |

303B: (*EXC  exit CASE statement*) DEC(S); PC := stk[S] |

304B: (*TRAP*)  i := pop(); Trap(i) |

305B: (*CHK*) k := pop(); j := pop(); i := pop(); push(i);

IF (INTEGER(i) < INTEGER(j)) OR

(INTEGER(i) > INTEGER(k)) THEN Trap(4) END  |

306B: (*CHKZ*)

k := pop(); i := pop(); push(i);

IF i>k THEN Trap(4) END  |

307B: (*CHKS  check sign bit*)

k := pop(); push(k);

IF INTEGER(k) < 0  THEN Trap(4) END |

310B: (*EQL*)  j := pop(); i := pop();

IF i = j THEN push(1) ELSE push(0) END  |

311B: (*NEQ*)  j := pop(); i := pop();

IF i # j THEN push(1) ELSE push(0) END  |

312B: (*LSS*) j := pop(); i := pop();

IF INTEGER(i) < INTEGER(j) THEN

push(1) ELSE push(0)

END  |

313B: (*LEQ*) j := pop(); i := pop();

IF INTEGER(i) <= INTEGER(j) THEN

push(1) ELSE push(0)

END |

314B: (*GTR*) j := pop(); i := pop();

IF INTEGER(i) > INTEGER(j) THEN

push(1) ELSE push(0)

END  |

315B: (*GEQ*) j := pop(); i := pop();

IF INTEGER(i) >= INTEGER(j) THEN

push(1) ELSE push(0)

END |

316B: (*ABS*)  push(ABS(INTEGER(pop())))  |

- 13 -

317B: (*NEG*)  push(-INTEGER(pop()))  |

320B: (*OR*)   j := pop(); i := pop();

push(CARDINAL(BITSET(i)+BITSET(j))) |

321B: (*XOR*)  j := pop(); i := pop();

push(CARDINAL(BITSET(i)/BITSET(j))) |

322B: (*AND*)  j := pop(); i := pop();

push(CARDINAL(BITSET(i)*BITSET(j))) |

323B: (*COM*)  push(CARDINAL({0..15}/BITSET(pop()))) |

324B: (*IN*)   j := pop(); i := pop();

IF i > 15 THEN push(0)

ELSIF i IN BITSET(j) THEN push(1)

ELSE push(0)

END |

325B: (*LIN  load immediate NIL*)   push(177777B) |

326B: (*MSK*)  j := pop() MOD 16; push(CARDINAL({0..k-1})) |

327B: (*NOT*)  i := pop();  push(CARDINAL({15}/{j})) |

330B: (*ADD*)  j := pop(); i := pop();

push(CARDINAL(INTEGER(i) + INTEGER(j))) |

331B: (*SUB*)  j := pop(); i := pop();

push(CARDINAL(INTEGER(i) - INTEGER(j))) |

332B: (*MUL*)  j := pop(); i := pop();

push(CARDINAL(INTEGER(i) * INTEGER(j))) |

333B: (*DIV*)  j := pop(); i := pop();

push(CARDINAL(INTEGER(i) DIV INTEGER(j))) |

334B: (*MOD*)  j := pop(); i := pop();

push(CARDINAL(INTEGER(i) MOD INTEGER(j))) |

335B: (*BIT*)  j := pop() MOD 16; (* k := {j} *) push(k) |

336B: (*NOP*) |

337B: (*MOVF  move frame *) i := pop();

j := pop()+pop()*4; (*18 bits*)

k := pop()+pop()*4; (*18 bits*)

WHILE i>0 DO

stk[k] := stk[j]; INC(k); INC(j); DEC(i)

END |

340B: (*MOV  move block*)

k := pop(); j := pop(); i := pop();

- 14 -

WHILE k>0 DO

stk[i] := stk[j]; INC(i); INC(j); DEC(k)

END |

341B: (*CMP  compare blocks*)

k := pop(); j := pop(); i := pop();

IF k=0 THEN push(0); push(0)

ELSE

WHILE(stk[i] # stk[j]) AND (k > 0) DO

INC(i); INC(j); DEC(k)

END;

push(stk[i]); push(stk[j])

END |

342B: (*DDT  display dot*)

k := pop(); j := pop(); dbmd := pop(); i := pop()

(* display point at <j,k> in mode i inside

bitmap dbmd *) |

343B: (*REPL  replicate pattern *)

db := pop(); sb := pop(); dbmd := pop(); i := pop()

(* replicate pattern sb over block db inside

bitmap dbmd in mode i *) |

344B: (*BBLT  bit block transfer*)

sbmd := pop(); db := pop(); sb := pop();

dbmd := pop(); i := pop()

(* transfer block sb in bitmap sbmd to block db

inside bitmap dbmd in mode i *) |

345B: (*DCH  display character*)

j := pop(); db := pop(); fo := pop(); dbmd := pop()

(* copy bit pattern for character j from font fo

to block db inside bitmap dbmd *) |

346B: (*UNPK  unpack*) k := pop(); j := pop(); i := pop();

(*extract bits i..j from k, then right adjust*)

push(k) |

347B: (*PACK  pack*)

k := pop(); j := pop(); i := pop(); adr := pop();

(*pack the rightmost j-i+1 bits of k into positions

i..j of word stk[adr] *) |

350B: (*GB  get base adr n levels down*)

i := L; j := next();

REPEAT

i := stk[i]; DEC(j)

UNTIL j=0;

push(i) |

351B: (*GB1  get base adr 1 level down*)  push(stk[L]) |

- 15 -

352B: (*ALLOC  allocate block*)

i := pop(); push(S); S := S + i;

IF S > H THEN S := pop(); Trap(3) END |

353B: (*ENTR  enter procedure*)

i := next();  S := S+i;

IF S > H THEN S := S-i; Trap(3) END |

354B: (*RTN  return from procedure*)

S := L; L := stk[S+1]; i := stk[S+2];

IF i < 100000B THEN PC := i

ELSE G := stk[S]; F := stk[G]; PC := i - 100000B

END |

355B: (*CX  call external procedure*)

j := next(); i := next();

mark(G, TRUE); G := stk[dft+j];

F := stk[G]; PC := 2*i; PC := next2()  |

356B: (*CI  call procedure at intermediate level*)

i := next(); mark(pop(), FALSE);

PC := 2*i; PC := next2()  |

357B: (*CF  call formal procedure*)

i := stk[S-1]; mark(G, TRUE);

j := i DIV 400B; G := stk[dft+j];

F := stk[G]; PC := 2*(i MOD 400B); PC := next2()  |

360B: (*CL  call local procedure*)

i := next(); mark(L, FALSE);

PC := 2*i; PC := next2()  |

361B .. 377B:  (*CL1 - CL15  call local procedure*)

mark(L, FALSE); PC := 2*(IR MOD 16); PC := next2()

END

END (*LOOP*)

END Interpreter.